User Manual

MiniMag Readers

Models IDMB

USB-HID KB, USB-HID, RS232/USBCDC or PS/2

jpos service object

Reference

06/21/2006

800xxxxx-001

Rev. A

#xxx

Copyright 2006, International Technologies & Systems Corporation. All rights reserved. ID TECH is a registered trademark of International Technologies & Systems Corporation. Value through Innovation, MiniMag is a trademark of International Technologies & Systems Corporation.

ID TECH SOFTWARE LICENSE AGREEMENT

ID TECH ("LICENSOR") IS WILLING TO LICENSE THIS SOFTWARE TO YOU ONLY IF YOU ACCEPT ALL OF THE TERMS IN THIS LICENSE AGREEMENT. PLEASE READ THE TERMS CAREFULLY BEFORE YOU AGREE BECAUSE YOU WILL BE BOUND BY THE TERMS OF THIS AGREEMENT. IF YOU DO NOT AGREE TO THESE TERMS, LICENSOR WILL NOT LICENSE THIS SOFTWARE TO YOU.

Ownership of the Software

1.  The Licensor software program ("Software") and any accompanying written materials are owned by Licensor [or its suppliers] and are protected by United States copyright laws, by laws of other nations, and by international treaties.

Grant of License

2.  Licensor grants the right to use the Software in conjunction with an ID TECH product. You may load one copy into permanent memory of one computer and may use that copy only on that same computer.

Restrictions on Use and Transfer

3.  The Software may not be copied, except that (1) one copy of the Software may be made solely for backup or archival purposes, and (2) the Software may be transfer to a single hard disk provided the original is kept solely for backup or archival purposes.  The written materials may not be copied.

4.  The Software may be permanently transferred and any accompanying written materials (including the most recent update and all prior versions) if no copies are retained and the transferee agrees to be bound by the terms of this Agreement.  Such a transfer terminates your license. The software may not be rented or leased or otherwise transferred or assigned the right to use the Software, except as stated in this paragraph.

5.  The software may not be reverse engineered, decompiled, or disassembled.

Limited Warranty

6.  If used in conjunction with an ID TECH product, Licensor warrants that the Software will perform substantially in accordance with the accompanying written materials for a period of 90 days from the date of your receipt of the Software. Any implied warranties on the Software are limited to 90 days. Some states and territories do not allow limitations on duration of an implied warranty, so the above limitation may not apply to you.

7.  LICENSOR DISCLAIMS ALL OTHER WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, AND NON-INFRINGEMENT, WITH RESPECT TO THE SOFTWARE AND ANY ACCOMPANYING WRITTEN MATERIALS. This limited warranty gives you specific legal rights. You may have others, which vary from state to state.

8.  LICENSOR'S ENTIRE LIABILITY AND YOUR EXCLUSIVE REMEDY SHALL BE REPLACEMENT OF THE SOFTWARE THAT DOES NOT MEET LICENSOR'S LIMITED WARRANTY. Any replacement Software will be warranted for the remainder of the original warranty period or 30 days, whichever is longer.

9.  This Limited Warranty is void if failure of the Software has resulted from modification, accident, abuse, or misapplication.

10.  IN NO EVENT WILL LICENSOR BE LIABLE TO YOU FOR DAMAGES, INCLUDING ANY LOSS OF PROFITS, LOST SAVINGS, OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO USE THE SOFTWARE. Because some states do not allow the exclusion or limitation of liability for consequential or incidental damages, the above limitation may not apply you.

11.  This Agreement is governed by the laws of the state of California.

12. For any questions concerning this Agreement or to contact Licensor for any reason, please write:  International Technologies & Systems Corporation, 10721 Walker Street, Cypress, CA 90630 or call (714) 761-6368.

13.  U.S. Government Restricted Rights. The Software and documentation are provided with Restricted Rights. Use, duplication, or disclosure by the Government is subject to restrictions set forth in subparagraph (c)(1) of The Rights in Technical Data and Computer Software clause at DFARS 252.227-7013 or subparagraphs (c)(1)(ii) and (2) of Commercial Computer Software - Restricted Rights at 48 CFR 52.227-19, as applicable. Supplier is ID TECH, 10721 Walker Street, Cypress, CA 90630.

Revision History

Table of Contents

| Revision Date | Description |
| --- | --- |
| 02/20/06 | Initial Draft |
| 02/28/06 | Add USB HID connector support |
| 03/01/06 | General edits |
| 05/17/06 | Add RS232/USBCDC connector support |
| 06/21/06 | General edit |
| 02/23/07 | Support JRE 1.4 |
| 12/21/07 | Support IBM JRE 1.4 |

Introduction	4

System Requirements:	4

Reference Document:	4

Environment Build	5

Run POSTest	8

A Sample for using MSR JPOS	15

# Introduction

This document provides information for using JPOS Service Object (SO) for MiniMag readers with model number IDMB.  Visit the ID TECH website at http://www.idtechproducts.com for additional information on readers.

# System Requirements:

This JPOS SO program is for Windows 98, Windows 2000 and Windows XP.

# Reference Document:

1. Unified POS Retail Peripheral Architecture Version 1.9 RC1 December 1, 2004

2. International Standard for Implementation of Point Of Service Peripherals

# Environment Build

Install SunJRE 1.4, 1.5 or JDK 5.0, IBM JRE 1.4:

Download the newest JRE 1.5 installation package from: http://java.sun.com/j2se/1.5.0/download.jsp, then click: Download JDK 5.0 Update  and select a proper platform JDK version to download. Install the package and make sure the "java" command functions properly. There may be a need to add the JRE bin directory into the system environments variables by following steps:

For MS Windows 2K/XP

1. right click "My Computer" in the desktop

2. select the "properties" menu item

3. select "Advanced" tab

4. click button "Environment Variables"

5. select "Path" and insert the JRE bin directory.

For MS Window 98

Add a line into to "Autoexec.bat" in the system directory:

PATH=%PATH%;JRE or JDK Bin Dir

Then reboot the system.

When Java is working, following can be observed:

Enter DOS command shell, and enter java. It shows information about how to use.

FILELISTS:

jmsrso.jar								JPOS device service control for Minimag II

jpos.xml									jpos configure file

jpos.properties							jpos configure file, this specifies jpos.xml

com_idtech_connector_JConnector.dll		JNI DLL extension

usbkbConnector.dll						extension DLL

PS2Conn.dll								Extension DLL for PS2 access

usbhidConn.dll							extension DLL to access usbhid

SerialConn.dll								extension DLL to access serial port or USB/CDC

IDTECH_KeyboardHook.dll					extension DLL

WinIO.dll									Extension DLL for PS2 access

WinIO.sys								Extension for PS2 access for Windows XP/2000

WinIO.vxd								Extension for PS2 access for Windows98

- The .DLL should be located in the directory in the PATH environment specified. For example, if the current working directory is included in the PATH variable,

PATH=%PATH%;.\

So, the DLLs can be moved to current directory.

NOTE On Windows 98

WinIO.dll, WinIO.sys, WinIO.vxd should be placed in the same directory, and add the directory to environment PATH variable in file autoexec.bat at Windows directory. For Example the directory the WinIO.xxx located is c:\jpos\ext\, a line as following should add to file autoexec.bat

set PATH=c:\jpos\ext;%PATH%

then reboot the Windows 98

Device Name: the jpos.xml the JPOS register file, that specifiy the device names.

There are several entries in this file, with entry name:

- “defaultMSR”

- “IDTECH_MMII_USBKB”

- “IDTECH_MMII_USBHID”

- “IDTECH_MMII_RS232”

- “IDTECH_MMII_PS2”

- “IDTECH_MMII_USBCDC”

“defaultMSR”

this entry is a copy of one of other entry as the default name for POSTest program.

“IDTECH_MMII_USBKB”,

The line: <creation factoryClass="com.idtech.jpos.IDTJposServiceInstanceFactory" serviceClass="com.idtech.MinimagService" /> specifies the factory class and service class in this implementation.

The line: <prop name="connector" type="String" value="usbkb/0acd/0520" /> specifies the connector driver name, and the configuration value. 0ACD is the ID TECH Vendor ID; 0520 is the product ID.

“IDTECH_MMII_USBHID”,

<prop name="connector" type="String" value="usbhid/0acd/0500"/>, 0ACD is the ID TECH Vendor ID; 0500 is the product ID.

“IDTECH_MMII_RS232”, “IDTECH_MMII_USBCDC”

This tow entries are same but the name. An additional COM port is presented if the USB CDC driver is installed.

The line:<prop name="connector" type="String" value="COM1/baud=9600/parity=N/data=8/stop=1"/> specify the serial port parameters. “COM1” stand for the port number. “baud=9600/parity=N/data=8/stop=1” means the serial port settings: baud rate 9600, none parity check, data bits are 8 bits, stop bits are 1 bit.

“IDTECH_MMII_PS2”,

<prop name="connector" type="String" value="ps2"/>

“ps2” is the all parameter.

- jpos/res/jpos.properties

This file specify which configure file is used. By default, it specify the “jpos.xml” as the one.

- Add jmsrso.jar to your CLASSPATH or specify it in the javac / java command.

set CLASSPATH=%CLASSPATH%;yourDir\jmsrso.jar

For example: jmsrso.jar is placed in c:\so,

it should set CLASSPATH=%CLASSPATH%;c:\so\jmsrso.jar

# Run POSTest

It's the official recommended JPOS compatible test program. The release version includes the program or it can be gotten from http://sourceforge.net/projects/postest/

1. unzip file "JPOS ServiceControl v01.000.zip" to a directory. Such as

c:\temp

The Directory is like the following:

Enter "c:\temp"Postest directory under the DOS command shell

Then enter subdirctory "POStest"

2. To run POSTest just like following:

Then a POStester program screen is shown.

3. Click "MSR" Tab

The logical name is the entry name in the jpos.xml file.

4. Click "Open" button, and "Claim", and three “beeps” should sound from the device.

5. Check choices "Data event enabled", "Device enabled"

6. Observe the text field and swipe a card.

7. Click "Release" or "Close" to release or close the JPOS,

NOTE: This should be done to make sure the device returns to normal state.

8. click "EXIT" to quit the program.

For more detail please check the http://www.javapos.com and the release help files.

# A Sample for using MSR JPOS

The following code is the sample:

import jpos.*;

import jpos.config.*;

import jpos.config.simple.*;

import jpos.loader.*;

import jpos.loader.simple.*;

import jpos.profile.*;

import jpos.services.*;

import jpos.util.*;

import jpos.util.tracing.*;

public class YourClass{

public static void main(String [] args){

MSR msr=new MSR(); /*  new an instance */

try{

msr.open(TestConst.testdevicename);

}catch(JposException e){

System.err.print(e);

}

/**

Do something here.

*/

try{

msr.close();

}catch(JposException e){

System.err.print(e);

}

}

}